Centering and Alignment Filters

Plug-in Filters for NIH-Image

Carl Gustafson and Oleh Tretiak

Autoradiographic Image Processing Center

an NIH  Biomedical Technology Resource

Drexel University, Philadelphia, PA

(215)895-2279

Comments to: carl.gustafson@ece.drexel.edu

The archive you should have received contains three files: this document, and two plug-in filters, one named CenterSlice and the other named AlignSlice. These filters are a result of some of the research into image processing carried out at the Computer Vision Center for Vertebrate Brain Mapping over the past several years.

The CenterSlice filter is used to center an image in it’s window. The image is both centered and rotated so that the major axis of the best-fit ellipse is horizontal.

The AlignSlice filter aligns two images, shifting and rotating the image in one window so that corresponds as well as possible to the image in the other, or reference, window. This is particularly useful with histology and autoradiogram images, for example.

These filters are designed to work only with NIH Image. Even though they are technically Photoshop filters, they (especially AlignSlice) make use of detailed knowledge of the window-based data structures used in NIH Image, and will probably crash and burn in other hosts. They specifically check to see if they are running on NIH Image, and should not even run with other programs.

Using the plug-ins

Copy the plug-ins to the Plug-ins folder inside the Image folder. They will automatically be available the next time you start NIH-Image. To start the filters going, just choose the correct item from the Plug-in Filters submenu of the Enhance menu. The filters ignore any ROI you may have selected.

To get the best results from these filters, you should be able to mark distinctly shaped areas of the image using the Density Slice option in Image. If the density slice you wish to center or align is diffuse or spread through the entire image area, you may not get useful results.

When using the AlignSlice filter, you will need to have two images open. The filter will align the image in the uppermost window with the one directly behind it (the reference window), which will be unchanged.

The filters require that you enter the starting and ending pixel values of the density slices for the areas you want to center/align. Using the Density Slice option, adjust your image so that the area of interest is red. Note the high and low values in the Values window. These are the numbers that you enter for density slice start and end in the filter dialog. You do not need identical slice start and end values when aligning two images.

How it works.

The alignment and registration techniques are based on the method of moments.  Moments were introduced to image pattern recognition by Hu in 1962, and the textbook by A. K. Jain provides a good discussion of this method.  In our implementation, a region is interpreted as a rigid body, as if it was cut out from sheet metal.  Moments up to second order are computed.  These, in effect, allow us to compute a best fit ellipse for the region, that is, an ellipse that has the same centroid and second order moments as the region in question.  In the centering option, the image is translated so that the centroid of the object is at the center of the window, and the long axis of the ellipse is horizontal.  This is illustrated in Figure 1.  In the alignment option, the best fit ellipses of two regions are computed.  The first window is translated so that its region's centroid coincides with that of the region in the second window, and is rotated about this centroid so that the major axes of the two best fit ellipses are parallel.

(a)	(b)

Figure 1.  (a)  A triangular region and its best-fit ellipse.  (b)  Same region, after centering.

We have performed extensive tests of this technique, and found that when it is good, it is very, very good.  When the regions are identical, such as when scanning the same high contrast object with different orientations and locations, the alignment error due to the

algorithms is typically less than half a pixel.  The magnitude of the rotational error is,  in effect, also less than a pixel.

Caveats

The algorithm can get into difficulty in two ways.  First, if the regions used for alignment have different shapes, then exact alignment is not possible.  This difficulty is understandable.  Judicious preprocessing may be used to overcome this problem.  The second source of difficulty is intrinsic to the method.  Since we are working with best-fit ellipses,  whenever the best fit ellipse is a circle there is no useful information for orientational alignment.  This may occur even with regions that contain sufficient information for alignment, as shown in Figure 2.  It is unfortunate, but the moments method does not generate the required angular alignment information for such objects.  This issue also is relevant when the best fit ellipse is almost circular, because then the alignment angle is defined, but is easily perturbed by small variations in region shape.

A more subtle version of the difficulty produced by the use of the best-fit ellipse is that the ellipse, when rotated by 180°, is unchanged.  Consequently, the algorithm cannot distinguish between, for example, 5° and –175° rotations.   The implementation we use always assumes that the rotation is between -90° and 90°.  Should the correct rotation be greater than ±90° the algorithm will produce a result that is off by 180°.   This can be fixed by rotating by 180° before the algorithm is applied.

Figure 2.  An arrow-shaped region, and its best-fit ellipse.  Note the ellipse happens to be circular, so that the algorithm will not be able to find the correct angular alignment for this object.

Acknowledgments

This algorithm was developed by the Autoradiographic Image Processing Center supported by NIH award P41RR01638, a Biomedical Technology Resource grant.

References

Ming-Kei Hu, Visual Pattern Recognition by Moment Invariants, IRE Trans. IT-8:179-187 (1962).

Anil K. Jain, Fundamentals of Digital Image Processing, Prentice Hall, Englewood Cliffs, NJ,  (1989).

Thanks

We would like to thank Wayne Rasband and especially Dietmar Reiter, of the Dept. of Zoology, Univ. Innsbruck, Austria, for their helpful feedback during testing of these filters.